ODM Truth vs Satellite Truth Comparison

Baseline root: /mnt/d/aiproject/imagematch/new2output/nadir_009010_dinov2_romav2_pose_2026-04-10

Current experiment root: /mnt/d/aiproject/imagematch/new3output/nadir_009010_dinov2_romav2_pose_odmrefresh_sattruth_2026-04-16

Runtime retrieval and candidate DOM library remained fixed to the satellite library.

Pose Runtime

| Run | PnP rows | PnP status counts | Best rows | Best status counts |
| --- | --- | --- | --- | --- |
| baseline | 800 | {"ok": 734, "pnp_failed": 66} | 40 | {"ok": 40} |
| current | 800 | {"ok": 520, "dsm_nodata_too_high": 217, "pnp_failed": 61, "dsm_coverage_insufficient": 2} | 40 | {"ok": 40} |

Validation Summary

| Suite | Metric | Value |
| --- | --- | --- |
| baseline_uav_truth | layer1 phase_corr_error_m mean | - |
| baseline_uav_truth | layer1 ortho_iou mean | - |
| baseline_uav_truth | layer1 ssim mean | - |
| baseline_uav_truth | layer2 horizontal_error_m mean | - |
| baseline_uav_truth | layer2 view_dir_angle_error_deg mean | - |
| baseline_uav_truth | layer3 tiepoint_xy_error_rmse_m | 2.8552 |
| baseline_uav_truth | layer3 tiepoint_xy_error_p90_m | 4.3476 |
| odm_truth_refresh | layer1 phase_corr_error_m mean | - |
| odm_truth_refresh | layer1 ortho_iou mean | - |
| odm_truth_refresh | layer1 ssim mean | - |
| odm_truth_refresh | layer2 horizontal_error_m mean | - |
| odm_truth_refresh | layer2 view_dir_angle_error_deg mean | - |
| odm_truth_refresh | layer3 tiepoint_xy_error_rmse_m | 2.3427 |
| odm_truth_refresh | layer3 tiepoint_xy_error_p90_m | 3.2604 |
| satellite_truth | layer1 phase_corr_error_m mean | - |
| satellite_truth | layer1 ssim mean | - |
| satellite_truth | layer2 truth_patch_center_offset_m mean | - |
| satellite_truth | layer2 truth_covering_tile_rank mean | - |
| satellite_truth | layer3 tiepoint_xy_error_rmse_m | 181.3715 |
| satellite_truth | layer3 tiepoint_xy_error_p90_m | 320.9374 |

Interpretation

ODM truth refresh keeps the UAV orthophoto-truth suite structure but replaces the truth orthophoto source and the PnP DSM source.

Satellite truth is an independent cross-check; its layer-2 is a geometry diagnostic against the truth patch rather than a pose-vs-AT metric.